CCAI: Final Skills Exam

Instructor: Tran Dac Tot

Topology

2019	. All rights reserved. This document is Cisco Public.

# Part 1: IPv4 (60 points)

## 1) Addressing Scheme

- HQ area is used network address X.X.X.X/A

- Branch area is used network address Y.Y.Y.Y/B

- Address table:

| Network | Address |
| --- | --- |
| R7  R6 | 202.0.100.0/30 |
| R7  R8 | 202.0.100.4/30 |
| R5  ACCESS | 202.0.100.8/30 |

- Host number requirement for VLAN HQ site

| VLAN | Name | Host Number |
| --- | --- | --- |
| 10 | TEAM1 | 500 |
| 20 | TEAM2 | 200 |
| 30 | TEAM3 | 100 |
| 40 | GUEST | 100 |
| 50 | SERVERS | 50 |
| 60 | Management(Native) | 50 |

- Host number requirement for Branch site

| Device |  | Interface | Host Number |
| --- | --- | --- | --- |
| R1 | Lo 0 |  | 200 |
| R1 | Lo 1 |  | 300 |
| R2 | Lo 0 |  | 100 |
| R2 | Lo 1 |  | 100 |
| R3 | Lo 0 |  | 200 |
| R3 | Lo 1 |  | 100 |

## 2) PPP Connections

- Configuration PPP connection between R7 and R6 router using PAP authentication

- Configuration PPP connection between R7 and R8 router using CHAP authentication

## 3) Routing

- Configure OSPF routing protocol in HQ site

- Configure static routing protocol in Branch site

- On R5, configure a default route to ACCESS router and propagate it to OSPF process

- On R5, configure static route (use summary route) to Branch site and propagate OSPF process in HQ site.

- Set passive on appropriate interface

## 4) Switching

- Switch S2 is VTP Server, remain switchs are VTP client

- Change spanning tree protocol to Rapid PVST+ mode

- S1 is root bridge for VLAN 10, 20, 30, 40. S2 is root bridge for remain VLANs

- Use network address of Management VLAN assign to switch

- Configure SSH on all switches

- Configure router-on-a-stick Inter-VLAN routing on R6 for VLAN 10, 20, 30, 40 and R8 for VLAN 50,

60.

- Configure EtherChannel with LACP protocol for all links between switch.

## 5) NAT and DHCP

- Configure NAT overload on Access allow private addresses of HQ and Branch site can access Internet.

- Setup an Web in Servers VLAN, configure port forwarding allow hosts from Internet can access the HTTP and HTTPS service.

- Setup DHCP server on R4 to assign IP address and other parameters to host in VLAN 10, 20, 30, 40 automatically.

## 6) Other requirements

- Create ACL not allow users on GUEST VLAN  access to all network of HQ and Branch but can use to the Internet.

- Create ACL allow only SERVERS VLAN can SSH to switches

# Part 2: IPv6 (20 points)

## 1) Address Scheme

- For the networks connect routers

| Network | Address |
| --- | --- |
| Access  R5 | 2001:ABBA:AAAA:1::/64 |
| R7  R6 | 2001:ABBA:CCCC:1::/64 |
| R7  R8 | 2001:ABBA:DDDD:1::/64 |
| R4  R5 | 2001:ABBA:EEEE:1::/64 |
| R4  R7 | 2001:ABBA:FFFF:1::/64 |

- Assign static link-local address for all interface using FE80::/10 range.

- Use the first five subnets of network 2001

- :ABBA:BBBB:/48 for five VLAN 10, 20, 30, 40, 50. The default gateway will be used the first IP address. Remark: assign appropriate static link-local address by yourself.

## 2) Routing

- Configure IPv6 static routing in HQ site.

- Configure a default route from R5 to Access and R5 is also default gateway for all remain networks

- Configure inter-VLAN routing using the same sub-interface for each VLAN in above IPv4  configuration.

## 3) DHCP

- Setup Stateless DHCPv6 on R7 router to assign IP address and other information for VLAN 10, 20,30 and 40 dynamically. Use 2001:4860:4860::8888 for DNS Server address.

- Configure relay agent on appropriate interfaces.

# Part 3: Q&A (20 points)

## Appendix

Address Table

| No. | Device/Host | IP Adsress | Subnet mask/Prefix | Default gateway |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| … |  |  |  |  |